# PhD candidate assignment – Mixed traffic flow

Problem:
Consider a single-lane road segment with a mix of human-driven vehicles (HDVs) and automated vehicles (AVs). You are given initial positions, speeds, and vehicle types. Use a scenario with 15 vehicles (initial conditions are in the companion csv file)

Dynamics:
Vehicle motion: x(t+1) = x(t) + v(t) Δt
HDVs follow: v(t+1) = v(t) + α(v_lead - v) + β(s - s*)
Parameters: α=0.5, β=0.2, s*=10 m, Δt=1 s
AVs are controlled subject to speed and acceleration limits.

Disturbance (pedestrian):
At time t=20 s and position xp=400 m, a pedestrian enters the road and affects traffic for 10 seconds. Model this as a temporary local constraint (e.g., speed reduction or stop). Detailed modelling is not required.

Tasks:
1. Define quantitative metrics for safety and efficiency.
2. Formulate a control problem, using optimisation if appropriate.
3. Propose and justify a solution approach.
4. Implement a simulation scenario.
5. Investigate the relevant trade-offs and critically discuss how your main modelling and methodological choices influence the results and conclusions.

Deliverables:

- Report (max 2 pages + 1 page figures)

- Maximum 10-minute presentation at the interview

Notes:

Some aspects of the problem are intentionally left unspecified. State and justify any additional assumptions needed to obtain a well-defined and physically meaningful model.
In presenting you solution, focus on clarity and reasoning. Simplicity is preferred.

Explicitly indicate how you use GenAI tools.